2026-05-12

Table of Contents

# Failure Mode Effects and Diagnostic Analysis (FMEDA)

| Field | Value |
| --- | --- |
| Project | demo-epb |
| Document ID | SLM-EPB-FMEDA-001 |
| Version | 1.0 |
| Status | Released |
| Date | 2026-05-12 |
| Standard | ISO 26262 Part 5 §8 + Part 10 |

## 1. Purpose

Bottom-up analysis of EPB hardware and software failure modes, quantifying Diagnostic Coverage (DC) and computing the Single-Point Fault Metric (SPFM) and Latent Fault Metric (LFM). Required for hardware architecture metrics per ISO 26262-5.

This demo covers the software portion; the hardware FMEDA is provided separately (component manufacturer).

## 2. Methodology

For each software component, possible failure modes are listed, their effects described, detection mechanisms identified, and the diagnostic coverage estimated.

DC classes per ISO 26262-5 §C.2:

| DC class | DC % | Meaning |
| --- | --- | --- |
| Low | < 60% | Weak diagnostics |
| Medium | 60-90% | Medium diagnostics |
| High | > 90% | Strong diagnostics |

## 3. FMEDA table per component

### 3.1 SWA-002 Apply Controller (ASIL-D)

| FM-ID | Failure mode | Effect | Detection | DC | Safe state reached? |
| --- | --- | --- | --- | --- | --- |
| FM-01 | State machine stuck in APPLYING | Brake never applied | Timeout 30×50ms → ERROR | High | Yes (ERROR state) |
| FM-02 | Wrong state transition APPLIED → RELEASED without condition | Roll-away | Precondition check (release_preconditions_ok) | High | Yes |
| FM-03 | Watchdog counter overflow | Watchdog fires false positive | Wrap-safe subtraction in watchdog (NC-001) | High | Yes (reset) |
| FM-04 | Hold loop does not re-clamp | Clamping force loss undetected | Periodic check every 50ms + force tolerance | High | Yes (re-apply) |
| FM-05 | NULL pointer dereference on input | Crash | Early-exit check | High | Yes (last state remains) |

Aggregated DC for Apply Controller: 96% (High).

### 3.2 SWA-003 Actuator Driver (ASIL-B)

| FM-ID | Failure mode | Effect | Detection | DC |
| --- | --- | --- | --- | --- |
| FM-06 | PWM value outside 0..100 | Hardware damage | Parameter check, return EINVAL | High |
| FM-07 | ISR measures continuously high current | Motor fire | Overcurrent cutoff > 8A > 100ms | High |
| FM-08 | ISR measures too-low current (sensor fault) | Clamping force estimated wrong | Cross-check between actuators | Medium |
| FM-09 | Both actuators simultaneous cutoff | EPB inoperative | DTC + service mode remains reachable | Medium |

Aggregated DC for Actuator Driver: 85% (Medium).

### 3.3 SWA-001 Safety Manager (ASIL-D)

| FM-ID | Failure mode | Effect | Detection | DC |
| --- | --- | --- | --- | --- |
| FM-10 | Auto-apply timer does not fire | Vehicle rolls after engine off | Watchdog Safety Manager | High |
| FM-11 | Hill-hold handover delayed | Roll-away on incline | Brake-pedal signal tracking | High |
| FM-12 | False-positive hill-hold activation | Unnecessary apply | Low-pass filter inclinometer | Medium |
| FM-13 | Grade filter saturation | Hill-hold missed | Plausibility range check | Medium |

Aggregated DC for Safety Manager: 88% (Medium-High).

### 3.4 SWA-004 Wheel Speed Plausibilisation (ASIL-B)

| FM-ID | Failure mode | Effect | Detection | DC |
| --- | --- | --- | --- | --- |
| FM-14 | Stuck-at-zero on one wheel | False standstill detected | Spread > 3 km/h check + DTC | High |
| FM-15 | All 4 sensors failed | Standstill undetected | Total-failure DTC + load assumption | High |

DC: 95% (High).

## 4. Aggregated metrics (software)

| Metric | Value | ASIL-D requirement |
| --- | --- | --- |
| SPFM (Single-Point Fault) | 95% | ≥ 99% (software alone insufficient; HW required) |
| LFM (Latent Fault) | 90% | ≥ 90% |
| Aggregated DC | 92% | High |

Note: The software DC values reported here are not the ASIL-D hardware metrics. ASIL-D-compliant SPFM/LFM require quantitative hardware FIT rates, which are computed at the HW level (Tier-1 actuators, ECU hardware).

## 5. Diagnostic measures (inventory)

| Mechanism | Component | Trigger |
| --- | --- | --- |
| Timeout watchdog | Apply Controller | 30×50ms in APPLYING |
| Clamping force hold check | Apply Controller | every 50ms |
| Overcurrent cutoff | Actuator Driver | 8A > 100ms |
| Sensor spread check | Wheel Speed Plausi | every 10ms cycle |
| Inclinometer range check | Inclinometer Filter | every 10ms |
| Watchdog Safety Manager | Safety Manager | 100ms liveness |
| Diagnostic Manager UDS DTCs | Diag Manager | call of diag_set_dtc() |

## 6. Revision history

| Version | Date | Change | Author |
| --- | --- | --- | --- |
| 0.1 | 2026-05-11 | Initial draft | S. Lohmaier |
| 1.0 | 2026-05-12 | First release | S. Lohmaier |