2026-05-12

Table of Contents

# Service Manual — Electric Parking Brake (EPB)

| Field | Value |
| --- | --- |
| Product | demo-epb EPB ECU |
| Version | 1.0 |
| Date | 2026-05-12 |
| Audience | Workshop technicians |

## 1. Tools

- OBD-II diagnostic tester with UDS support (ISO 14229)

- Torque wrench 60 Nm

- Sliding tool 28×40 mm (for brake-pad replacement)

## 2. UDS diagnostics

### 2.1 Identification

| Parameter | Value |
| --- | --- |
| Tester address | 0x712 |
| ECU response | 0x71A |
| CAN baud rate | 500 kbit/s |

### 2.2 Service IDs

| SID | Service | Notes |
| --- | --- | --- |
| 0x10 | DiagnosticSessionControl | 0x03 = Extended Session |
| 0x11 | ECUReset | 0x01 = Hard Reset |
| 0x14 | ClearDiagnosticInformation | Clears all DTCs |
| 0x19 | ReadDTCInformation | Sub 0x02 = reportDTCByStatusMask |
| 0x22 | ReadDataByIdentifier | See DID list |
| 0x27 | SecurityAccess | Not implemented in demo |
| 0x31 | RoutineControl | 0x0301 = Service mode |

### 2.3 DIDs (Data Identifiers)

| DID | Description | Type |
| --- | --- | --- |
| 0xF187 | SW version | ASCII 16 byte |
| 0xF18B | ECU hardware version | ASCII 16 byte |
| 0x0301 | Clamping force left | uint16 (N) |
| 0x0302 | Clamping force right | uint16 (N) |
| 0x0303 | Motor current left | uint16 (mA) |
| 0x0304 | Motor current right | uint16 (mA) |
| 0x0305 | Inclinometer (filtered) | int16 (m°) |

## 3. DTC list

| DTC | Meaning | Action |
| --- | --- | --- |
| P0571 | EPB switch plausibility | Check switch |
| P0572 | EPB switch permanently actuated | Switch jammed? Clean |
| P0808 | Actuator current left too high (overcurrent) | Check motor + wiring |
| P0809 | Actuator current right too high (overcurrent) | Check motor + wiring |
| P080A | Clamping force left not reached (apply timeout) | Check actuator / mechanism |
| P080B | Clamping force right not reached | Check actuator / mechanism |
| P080C | Wheel-speed sensor plausibility | Check sensors / wiring |
| P080D | Inclinometer plausibility | Check sensor / mounting |
| P080E | Apply controller watchdog trip | Software reset; if recurring replace ECU |
| U0123 | CAN bus communication lost | Check CAN wiring + BCM status |

## 4. Service mode (brake-pad replacement)

### 4.1 Activation

Preconditions: - Ignition on, engine off - Vehicle on lift or with chocked wheels - Driver door closed (or door signal bypassed)

Steps: 1. Connect diagnostic tester, Extended Session (0x10 0x03) 2. Send RoutineControl 0x31 01 03 01 — start routine 3. ECU acknowledges, EPB LED starts blinking at 2 Hz 4. Actuators move to maintenance position (fully released)

### 4.2 Deactivation

- Send RoutineControl 0x31 02 03 01 — stop routine

- EPB LED stops blinking

- Apply function available again

### 4.3 Brake-pad replacement procedure

- Activate service mode (see above)

- Remove brake calliper

- Replace pads, grease guides

- Tighten calliper to 60 Nm

- Deactivate service mode

- Perform three apply/release cycles (bedding-in)

- Clear DTC memory (service 0x14)

## 5. Sensor check

### 5.1 Wheel-speed sensors

- Resistance: 800-1500 Ω at 20 °C

- Voltage at 50 km/h: 2-5 V peak-to-peak (Hall)

### 5.2 Inclinometer

- SPI bus 1 MHz

- Expected value on level road: 0 ± 0.5°

- Drift check: ECU + tester, monitor > 5 min

## 6. Actuator check

| Parameter | Target value |
| --- | --- |
| Resistance per motor | 0.8 – 1.2 Ω |
| Nominal current | 3 – 5 A |
| Peak current (apply) | 15 – 25 A |
| Cutoff threshold | 8 A for 100 ms |

## 7. Software update

- UDS Extended Session (0x10 0x03)

- Programming Session (0x10 0x02)

- Flashloader sequence per OEM specification

- Verify new SW version via DID 0xF187

## 8. Revision history

| Version | Date | Change | Author |
| --- | --- | --- | --- |
| 1.0 | 2026-05-12 | First release | S. Lohmaier |