2026-05-12

Table of Contents

# User Manual — Electric Parking Brake (EPB)

| Field | Value |
| --- | --- |
| Product | demo-epb EPB ECU |
| Version | 1.0 |
| Date | 2026-05-12 |
| Audience | Vehicle drivers |

Read the important safety information first! Familiarise yourself with the functions before using the EPB.

## 1. What is the Electric Parking Brake?

The Electric Parking Brake (EPB) replaces the classical handbrake. You operate it via a switch in the centre console; the system clamps the rear brakes electromechanically.

## 2. Operation

### 2.1 Engage the parking brake (apply)

- Bring the vehicle to a complete standstill.

- Keep the brake pedal pressed.

- Pull the EPB switch upwards (arrow points to the windshield).

- The red LED on the switch lights up steadily.

You will hear a soft humming sound — that is the actuator motors.

### 2.2 Release the parking brake

Preconditions (all must be met):

- Engine is running

- Brake pedal is pressed

- Gear selector is engaged (not in neutral)

- Push the EPB switch downwards.

- The LED goes out.

- You will hear a short humming sound again.

### 2.3 Auto-Hold (driver leaving the car)

When you switch the engine off and the vehicle is at a standstill, the EPB engages automatically after 2 seconds — even if you didn’t operate it manually. The LED confirms.

### 2.4 Hill-Hold on inclines

When stopping on a slope (> 5%):

- Press the brake pedal — vehicle stops.

- Lift your foot off the brake pedal — the EPB takes over automatically.

- The LED blinks slowly while hill-hold is active.

- On drive-away (throttle + gear engaged), the EPB releases automatically.

## 3. LED indicator meaning

| LED status | Meaning |
| --- | --- |
| Off | EPB released |
| Steady red | EPB active (apply / hold) |
| Slow blink (2 Hz) | Hill-hold active or service mode |
| Fast blink (4 Hz) | Fault — visit a workshop |

## 4. Display in the instrument cluster

The instrument cluster shows additional text:

| Text | Meaning |
| --- | --- |
| “EPB active” | Parking brake engaged |
| “Hill-Hold active” | Hill-hold is taking over |
| “EPB fault” | Fault — visit a workshop |
| “EPB service mode” | In workshop mode, do not release yourself |

## 5. Emergency mode

If the EPB does not respond:

- Stationary and won’t move: push the EPB switch downwards several times; if unsuccessful, call breakdown service.

- Stationary and the EPB does not engage: secure the vehicle with wheel chocks, contact a workshop.

## 6. Safety information

⚠ WARNING

- The EPB does not replace engaging a gear when parking

- On slippery surfaces additionally use wheel chocks

- While the engine is running and the EPB is engaged, do not stand on the brake pedal long-term

## 7. Maintenance

The EPB is maintenance-free. For brake pad replacement, the workshop must activate service mode — please do not jack up the vehicle yourself while the EPB is in the active state.

## 8. Revision history

| Version | Date | Change | Author |
| --- | --- | --- | --- |
| 1.0 | 2026-05-12 | First release | S. Lohmaier |