Table of Contents

# Project Initiation Document (PID)

| Field | Value |
| --- | --- |
| Project | demo-epb (Electric Parking Brake) |
| Project ID | SLM-EPB-001 |
| Client | slohmaier.com (in-house demo) |
| Contractor | Stefan Lohmaier |
| Date | 2026-05-11 |
| Version | 1.0 |
| Status | Released |
| Classification | Public |

## 1. Project purpose

Demonstration of the slohmaier Dev Process using an EPB ECU software. The goal is not the productive software but a complete demonstration of:

- ASPICE 4.0-compliant development flow

- ISO 26262-compliant handling of safety requirements (ASIL-D / ASIL-B / QM)

- MISRA C compliance

- Toolchain: Gitea + Doorstop + Cppcheck + gcov + CppUTest + pandoc

The target audience is potential customers who want to see what a real audit-ready engineering snapshot looks like.

## 2. Product description

An Electric Parking Brake (EPB) clamps two rear callipers via small electric motors at standstill and releases them on drive-away. Functional scope:

- Apply / Release on driver request

- Hold function with auto-apply on engine-off

- Drive-Away-Assist (auto-release on drive-away)

- Hill-Hold on inclines

- Actuator current monitoring

- Service mode for the workshop

- UDS diagnostics via CAN

## 3. Safety goals

| ID | Safety goal | ASIL |
| --- | --- | --- |
| SG-01 | Prevent unintended vehicle roll-away | D |
| SG-02 | Prevent unintended release of the parking brake | D |
| SG-03 | Prevent motor damage from overload | B |

Safety goals are detailed further in the system requirements (reqs/sys/).

## 4. Stakeholders

| Role | Person / Function |
| --- | --- |
| Project Owner | Stefan Lohmaier |
| Technical Lead | Stefan Lohmaier |
| Quality Assurance | Stefan Lohmaier |
| Reviewer | External reviewer (TBD) |
| Customer (demo) | Prospects / interested parties |

In a real project QA and TL would be separate persons; in this demo the role separation is kept on paper.

## 5. Deliverables

| Artefact | Format | Status |
| --- | --- | --- |
| PID, PM Plan, QA Plan, SWE Plan, Test Plan | Word | Available |
| System Requirements (SYS-001..010) | Doorstop MD | Available |
| Software Requirements (SWE-001..025) | Doorstop MD | Available |
| System Architecture (SA-001..005) | Doorstop MD | Available |
| Software Architecture (SWA-001..010) | Doorstop MD | Available |
| Source code (3 demo components) | C99 | Available |
| Unit tests + coverage report | CppUTest, lcov | Available |
| MISRA report | Cppcheck XML | Available |
| Traceability matrix | Doorstop HTML | Generated in CI |
| Review minutes (example) | Word | Available |
| MISRA Deviation Record (example) | Word | Available |

## 6. Schedule

Demo project, single-sprint creation. One-day initial creation, maintenance thereafter.

| Phase | Start | End |
| --- | --- | --- |
| Concept + setup | 2026-05-11 | 2026-05-11 |
| Requirements + architecture | 2026-05-11 | 2026-05-11 |
| Implementation of demo components | 2026-05-11 | 2026-05-11 |
| Tests + CI | 2026-05-11 | 2026-05-11 |
| Release v1.0 | 2026-05-11 | 2026-05-11 |

## 7. Budget

Demo project, no external budget. Internal effort.

## 8. Risks

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Demo is mistaken for production-ready code | M | M | Disclaimer in README + plain labelling |
| MISRA tooling update breaks CI | L | M | Pin tool versions in CI |
| Reviewer availability | M | L | Self-review documented (demo only) |

## 9. Success criteria

- All 35 requirements are linked and covered by architecture

- doorstop check is green

- MISRA check in CI is green (with documented deviations)

- Demo-component coverage meets target (see SWE Plan)

- The guided tour in the README is navigable by a prospect in < 30 min