2026-05-12

Table of Contents

# Hazard Analysis & Risk Assessment (HARA)

| Field | Value |
| --- | --- |
| Project | demo-epb (Electric Parking Brake) |
| Document ID | SLM-EPB-HARA-001 |
| Date | 2026-05-12 |
| Version | 1.0 |
| Status | Released |
| Standard | ISO 26262 Part 3 (Concept Phase) |
| Author | Stefan Lohmaier |
| Reviewer | (Tech Lead, independent in real project) |
| Approver | (Safety Manager, independent in real project) |

## 1. Purpose

Identification and classification of all relevant EPB hazards per ISO 26262-3. From the hazards, safety goals are derived and an Automotive Safety Integrity Level (ASIL) is assigned.

## 2. Item definition

The EPB is an electromechanical system that clamps both rear callipers using two small electric motors and releases them. Item boundary (ISO 26262-3 §5):

- Inside: EPB ECU, both calliper motors, EPB switch, status LED

- Outside: ESP, engine management, brake system (hydraulic), steering

- Interfaces: CAN bus, wheel-speed sensors, inclinometer

## 3. Operational situations & hazards

The following operational situations and hazards were identified in the concept workshop (2026-05-11):

### 3.1 Hazard list

| H-ID | Hazard | Operational situation |
| --- | --- | --- |
| H-01 | Unintended release of the parking brake at standstill | Vehicle parked on incline, driver out |
| H-02 | Unintended clamping during driving | Driving > 10 km/h |
| H-03 | No apply reaction to driver request | Standstill, driver actuates switch |
| H-04 | Loss of clamping force in hold state | Parking phase longer than 1 h |
| H-05 | Motor damage from overcurrent | Actuator mechanics blocked |
| H-06 | Incorrect hill-hold handover (roll-away on incline) | Drive-away on incline |
| H-07 | No release reaction on drive-away | Standstill, driver wants to drive |
| H-08 | LED indicator wrong | any |

### 3.2 Severity / Exposure / Controllability

Classification per ISO 26262-3 §6:

| Severity | Meaning |
| --- | --- |
| S0 | No injuries |
| S1 | Light / moderate injuries |
| S2 | Severe injuries (survival likely) |
| S3 | Life-threatening injuries (survival uncertain) |

| Exposure | Meaning |
| --- | --- |
| E0 | Very unlikely |
| E1 | Very rare situation |
| E2 | Rare situation |
| E3 | Medium likelihood |
| E4 | Frequent situation |

| Controllability | Meaning |
| --- | --- |
| C0 | Generally controllable |
| C1 | Simply controllable (>99% of drivers) |
| C2 | Normally controllable (>90% of drivers) |
| C3 | Difficult to control or uncontrollable |

### 3.3 ASIL determination

| H-ID | Description | S | E | C | ASIL |
| --- | --- | --- | --- | --- | --- |
| H-01 | Unintended release, parking phase | S3 | E4 | C3 | D |
| H-02 | Unintended clamping during driving | S3 | E4 | C3 | D |
| H-03 | No apply reaction to request | S2 | E4 | C2 | B |
| H-04 | Clamping force loss in hold | S3 | E4 | C3 | D |
| H-05 | Motor damage from overcurrent | S1 | E3 | C2 | A |
| H-06 | Hill-hold failure (roll-away on incline) | S3 | E3 | C3 | C |
| H-07 | No release reaction | S1 | E4 | C2 | A |
| H-08 | LED indicator wrong | S0 | – | – | QM |

ASIL matrix per ISO 26262-3 Table 4 applied. H-06 was downgraded from ASIL-D to ASIL-C in review, since hill-hold failure on dry road remains controllable through driver response (C2-C3 borderline, conservatively C3).

## 4. Safety goals

From the hazards the following safety goals are derived:

| SG-ID | Safety goal | ASIL | Covered hazards |
| --- | --- | --- | --- |
| SG-01 | The EPB must not unintentionally release while at standstill | D | H-01, H-04 |
| SG-02 | The EPB must not unintentionally clamp while driving | D | H-02 |
| SG-03 | The EPB must protect against actuator overcurrent | A | H-05 |
| SG-04 | Hill-hold must reliably hand over to the apply controller | C | H-06 |
| SG-05 | The EPB must respond to driver requests within specified times | B | H-03, H-07 |

## 5. Safe state

Definitions per ISO 26262-3 §7.4.2.5:

| Item / Function | Safe state |
| --- | --- |
| Apply phase | Stop actuator, set status to APPLIED |
| Hold phase | Maintain clamping force (passive) |
| Release phase | Return to apply, maintain clamping force |
| On hardware fault | Force APPLIED state (prevents roll-away) |

The conservative safe state across all cases is APPLIED: rather over-clamp than under-clamp.

## 6. FTTI (Fault Tolerant Time Interval)

| Hazard | FTTI | Rationale |
| --- | --- | --- |
| H-01 | 5 s | Roll-away on incline starts after ~1-2 s, hand action possible after ~5 s |
| H-02 | 100 ms | Shock deceleration at 50 km/h must be detected within 100 ms |
| H-04 | 30 s | Clamping force loss accumulates slowly, periodic check every 50 ms suffices |
| H-06 | 500 ms | Hill-hold handover must complete before roll-away begins (< 500 ms) |

## 7. Functional Safety Requirements (FSR)

From the safety goals the SYS requirements in reqs/sys/ are derived (see traceability matrix). Mapping:

| SG-ID | SYS requirements |
| --- | --- |
| SG-01 | SYS-001, SYS-004 |
| SG-02 | SYS-002 (apply plausibility), SYS-005 |
| SG-03 | SYS-007 |
| SG-04 | SYS-005, SYS-006 |
| SG-05 | SYS-002, SYS-003 |

## 8. Revision history

| Version | Date | Change | Author |
| --- | --- | --- | --- |
| 0.1 | 2026-05-11 | Initial draft | S. Lohmaier |
| 1.0 | 2026-05-12 | First release after review | S. Lohmaier |