2026-05-11

Table of Contents

# MISRA Deviation Record MISRA-REC-001

| Field | Value |
| --- | --- |
| Record ID | MISRA-REC-001 |
| Date | 2026-05-11 |
| File | src/apply_controller.c |
| Function | apply_ctrl_step_50ms |
| Line | 64 |
| Standard | MISRA C:2012 |
| Rule | Rule 15.5 (Advisory) — “A function should have a single point of exit” |
| ASIL | D |
| Status | Approved |

## 1. Code excerpt

void apply_ctrl_step_50ms(const ApplyInputs* in)
{
    if (in == NULL) {
        s_ctx.last_error = EPB_EINVAL;
        return;            /* <-- early exit */
    }
    ...
}

## 2. Rationale

The NULL pointer check as an early exit significantly improves readability versus a nested variant with a single return at the end. MISRA Rule 15.5 is Advisory, not Required.

The early exit has clearly defined semantics (input validation) and does not impair verifiability; on the contrary, the separate path is unambiguously covered in the unit test test_null_input.

## 3. Alternative considered

Single-exit variant:

void apply_ctrl_step_50ms(const ApplyInputs* in)
{
    if (in == NULL) {
        s_ctx.last_error = EPB_EINVAL;
    } else {
        /* entire step logic nested in else branch */
        ...
    }
}

Rejected because the additional nesting makes the state machine harder to read without gaining functional equivalence relative to the early-exit variant.

## 4. Safety impact

None. The early exit is deterministic and covered by the unit test.

## 5. Approval

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Technical Lead | Stefan Lohmaier | 2026-05-11 | (demo) |
| Safety Manager | (in real project) | 2026-05-11 | (demo) |

## 6. Scope

This deviation applies only to this specific code site. Other early-exit sites require separate records.