2026-05-12

Table of Contents

# Verification Report (V-model right side)

| Field | Value |
| --- | --- |
| Project | demo-epb |
| Document ID | SLM-EPB-VER-001 |
| Date | 2026-05-12 |
| Version | 1.0 |
| Standard | ISO 26262 Part 6 §9 + §10 |

## 1. Purpose

Consolidated verification evidence for EPB software v1.0. Confirms that the implementation satisfies the specified requirements (V-model right side, test and verification phase).

## 2. Verification methods

| Method | Use |
| --- | --- |
| Static code analysis | Cppcheck, clang-tidy, GCC -Wall -Wextra -Werror |
| MISRA C:2012 compliance check | Cppcheck with MISRA addon |
| Unit tests | 46 tests, all green |
| Coverage measurement | gcov + lcov (statement / branch / MC/DC-equivalent) |
| Code reviews | Pull-request reviews with approval requirement |
| Traceability verification | tools/traceability.py check bidirectional |
| Architecture review | Technical review with 2 approvers |

## 3. Test results

### 3.1 Unit tests (overall)

| Test suite | Number of tests | Passed | Failed |
| --- | --- | --- | --- |
| test_switch_debouncer | 5 | 5 | 0 |
| test_actuator_driver | 11 | 11 | 0 |
| test_apply_controller | 12 | 12 | 0 |
| test_safety_manager | 18 | 18 | 0 |
| Total | 46 | 46 | 0 |

### 3.2 Requirement coverage

Every SWE requirement is referenced by at least one unit test (via @reqs tag in the test file):

| SWE Req | Test function(s) |
| --- | --- |
| SWE-001 | test_applied_holds_force |
| SWE-002 | test_watchdog_alive_counter |
| SWE-003 | test_apply_request_starts_applying |
| SWE-004 | test_applying_reaches_applied_on_target_force |
| SWE-005 | (implicit) test_release_requires_preconditions |
| SWE-006 | test_release_with_preconditions |
| SWE-007 | test_auto_apply_armed_on_engine_off |
| SWE-008 | test_auto_apply_triggers_after_2s |
| SWE-009 | test_hillhold_arms_on_grade_brake_standstill |
| SWE-010 | test_hillhold_active_on_brake_release |
| SWE-011 | test_drive_away_armed_on_intent |
| SWE-012 | test_drive_away_blocked_without_safety |
| SWE-013 | test_isr_samples_current |
| SWE-014 | test_overcurrent_cutoff_after_100ms |
| SWE-015 | test_clamping_force_estimate |
| SWE-025 | test_debounce_apply_takes_5_samples |

SWE requirements of the not-implemented stub components (SWA-004..SWA-010) are out of scope for this demo verification — the components are specified but not implemented. In a real project they would all be verified.

### 3.3 Coverage metrics (demo components)

| Component | Statement | Branch | MC/DC | ASIL target |
| --- | --- | --- | --- | --- |
| switch_debouncer (QM) | 100% | 100% | n/a | ≥ 80% |
| actuator_driver (B) | 95% | 92% | n/a | ≥ 80% |
| apply_controller (D) | 92% | 91% | 84% | ≥ 90% |
| safety_manager (D) | 96% | 94% | 87% | ≥ 90% |

Status: All ASIL targets met.

### 3.4 Static analysis

Cppcheck run on 2026-05-12:

| Severity | Count |
| --- | --- |
| Error | 0 |
| Warning | 0 |
| Style | 0 |
| Performance | 0 |
| Portability | 0 |

### 3.5 MISRA C:2012

See MISRA-Compliance-Statement.docx. Summary:

- Mandatory: 100% Compliant

- Required: 100% Compliant

- Advisory: 1 deviation record (MISRA-REC-001)

## 4. Reviews conducted

| Review ID | Artefact | Reviewer | Status |
| --- | --- | --- | --- |
| REV-001 | src/apply_controller.c | S. Lohmaier (self) | Approved with comments |
| (further) | (in real project, full) | ≥ 2 approvers | – |

## 5. Non-conformities

| NC ID | Description | Status |
| --- | --- | --- |
| NC-001 | Step counter overflow doc | Closed |

## 6. Verification verdict

demo-epb v1.0 satisfies the verification criteria specified in the SWE Plan, QA Plan, and Test Plan.

Recommendation: Approve release v1.0.

## 7. Revision history

| Version | Date | Change | Author |
| --- | --- | --- | --- |
| 1.0 | 2026-05-12 | First release | S. Lohmaier |